################################################################################

Title:        RTL8139(X) PXE/RPL Boot ROM Program

History:      V1.00  --   1999.05.13

Notes:

#######################################################################################################

Name:          PXE-RPL.212

Version:       V2.12

PXE  Version:	 V2.12 (build 082)

RPL  Version:  V3.16

Released By:   Season Chen

Release Date:  2001.8.20

Changed File:

Reason:	1.Update RPL code from v3.15 to v3.16.

a.Update RTSODI.ASM from v3.11 to v3.64.

b.remove HOME key.

c.Reverse the Class code of PCI and PnP Expansion header

from 00 00 02 to 02 00 00.

2.Split the stub.r to romconf.r to reach the goal

of only 4K code will be left at HIMEM.

3.Update code to not change the monitor display mode.

4.Not extend the .nic rom to the 64K size. Copy the RPL

image from the Flash not from the Himem.(Use Big real

mode copy, do not use INT 15h service).

5.Modify code to be able to been compiled to merged image

and the split image.

6.Reverse the Class Code of PCI and PNP Expansion structure

from 00 00 02 to 02 00 00. And change the Indicator byte

of the PNP Expansion header from 14h to E4h.

7.Change PXE support Chip name to RTL8139(X)/8130/810X.

8.When searching the ROM BASE ADDRESS, make sure to avoid

the ROM_BASE and ROM_LIMIT of PCI-PCI bridge. Fix problem of

!PXE structure not found as TNT2 card is pluged in the PCI-PCI

bridge(COMPAQ case).

#######################################################################################################

Name:          PXE-RPL.213

Version:       V2.13

PXE  Version:	 V2.13 (build 082)

RPL  Version:  V3.16

Released By:   Season Chen

Release Date:  2002.3.26

Changed File:

Reason:	1.Always set chip to D0 power state.

2.Always turn on chip analog power.

3.Showing "Press shift-F10 to config" string.

4.Fix delay 2 minutes problem when EEPROM pxe

parameter is set to an unexpected value.

#######################################################################################################

Name:          PXE-RPL.214

Version:       V2.14

PXE  Version:	 no change

RPL  Version:  V3.17

Released By:   Season Chen

Release Date:  2004.3.5

Changed File:

Reason:

#######################################################################################################

Name:          PXE-RPL.215

Version:       V2.15

PXE  Version:	V2.15(build 082)

RPL  Version:	V3.17

Released By:	Leo Kuo

Release Date:  2004.7.29

Changed File:  1. Fix Interrupt handler routine entering into infinite loop 			  state.

Reason:

#######################################################################################################

Name:          PXE-RPL.216

Version:       V2.16

PXE  Version:	V2.16(build 082)

RPL  Version:	V3.17

Released By:	Leo Kuo

Release Date:  2004.12.14

Changed File:  1. The size of base memory is increased by 2K.

2.	Fixed TFTP Request sent to Gateway problem.(See bootp.c 					    GETBOOTINFO function)

Reason:

################################################################################

Name:          PXE-RPL.217

Version:       V2.17

PXE  Version:	V2.17(build 082)

RPL  Version:	V3.17

Released By:	Leo Kuo

Release Date:  2005.12.02

Changed File:  1.	Fixed TFTP Request sent to Gateway problem.(See bootp.c 					    Wait_For_DHCP_Ack function)

2. Change Show Message time to 3,5,1,0 second.

Reason:

################################################################################

Name:          PXE-RPL.218

Version:       V2.18

PXE  Version:	V2.18(build 082)

RPL  Version:	V3.17

Released By:	Leo Kuo

Release Date:  2006.03.27

Changed File:  nic.asm

Reason: 	1. Wait DMA complete to transmit and receive packet.

2. Increase the base memory only to 1KB instead of 2KB

################################################################################

Name:          PXE-RPL.220

Version:       V2.20

PXE  Version:	V2.20(build 082)

RPL  Version:	V3.18

Released By:	Leo Kuo

Release Date:  2006.07.11

Changed File:  nic.asm

Reason: 	1.  In Resetnic routine, write 0 in register MAR0-MAR7.

2. Fix share interrupt hang problem with SATA HD in RPL.

##################################################################